ESEIAAT - UPC

Study for the computational resolution of conservation equations of mass, momentum and energy. Possible application to different aeronautical and industrial engineering problems: Case 1B

Report

Laura Pla Olea

10/06/2017

[Escriba aquí una descripción breve del documento. Normalmente, una descripción breve es un resumen corto del contenido del documento. Escriba aquí una descripción breve del documento. Normalmente, una descripción breve es un resumen corto del contenido del documento.]

- Content

1	Introduction	4

2	State of the art	5

2.1	Conservation equations	5

2.2	Numerical methods	5

2.2.1	Finite difference method	6

2.2.2	Finite volume method	6

2.3	Time integration	7

2.4	Evaluation of the convective term	8

2.4.1	The central differencing scheme (CDS)	8

2.4.2	The upwind scheme (UDS)	8

2.4.3	The exponential scheme (EDS)	8

2.4.4	The hybrid scheme (HDS) and the power-law scheme (PLDS)	9

3	Four materials	10

3.1	Conduction equation	11

3.2	Discretization	11

3.2.1	Space discretization	11

3.2.2	Time discretization	12

3.2.3	Boundary conditions	14

3.3	Algorithm	16

3.4	Results	18

4	Smith-Hutton problem	19

4.1	General convection-diffusion equation	19

4.2	Discretization	20

4.3	Boundary conditions	24

4.4	Results	24

Figure 1: Three successive grid points	6

Figure 2: Control volume (2D)	7

Figure 3: Explicit method (β=0)	7

Figure 4: Implicit method (β=1)	7

Figure 5: Crank-Nicholson (β=0,5)	7

Figure 6: General scheme of the four materials problem	10

Figure 7: Heat fluxes through the faces of a control volume	11

Figure 8: Instantaneous isotherms at t=5000 s	18

Figure 9: Instantaneous results at t=10000 s	18

Figure 10: General scheme of the Smith-Hutton problem	19

Figure 11: Control volumen (2D)	¡Error! Marcador no definido.

Figure 12: ρ/Γ=10	25

Figure 13: ρ/Γ=103	25

Figure 14: ρ/Γ=106	26

Table 1: Particular cases of the convection-diffusion equation	5

Table 2: Problem coordinates	10

Table 3: Physical properties of the materials	10

Table 4: Boundary conditions	10

Table 5: Function AP for different schemes1	23

# Introduction

# State of the art

The laws governing the processes of heat transfer and fluid flow are usually expressed in terms of differential equations. Some of these equations are the momentum equation, the energy equation and the mass conservation equation, among others. However, these expressions usually don’t have an analytical solution except for some simple cases. To solve complex problems it is necessary to use numerical methods.

## Conservation equations

The three most important conservation equations are the mass conservation equation, the momentum conservation equation and the energy conservation equation:

∂ρ∂t+∇·ρv=0

∂∂tρv+∇·ρvv=-∇p+∇·τ+ρg+fe

∂∂tρu+∇·ρvu=-∇·q-p∇·v+τ:∇v+Φe

For incompressible flows with no viscous dissipation, the energy conservation equation can be written as:

ρcp∂T∂t+v·∇T=∇·λ∇T

All these equations can be seen as a particular case of the generic convection-diffusion equation:

∂ρϕ∂t+∇·ρvϕ=∇·Γ∇ϕ+Sϕ

Where ρ is the density, v the velocity, Γ the diffusion coefficient, Sϕ the source term, and ϕ the general variable that is going to be studied.

| Equation | φ | Γ | Sφ |
| --- | --- | --- | --- |
| Mass conservation | 1 | 0 | 0 |
| Momentum | v | μ | -∇p+∇·τ-∇·μ∇v+ρg |
| Energy (for a semi perfect gas) | u | λ/cv | -∇·qn-p∇·v+τ:∇v |

Table 1: Particular cases of the convection-diffusion equation

## Numerical methods

Numerical methods are based in dividing the domain that is going to be studied in different pieces. Instead of calculating the unknowns in the whole domain, they are studied in the finite number of points defined by these pieces, the grid points. This process is called discretization.

Once the domain is discretized, it is also necessary to discretize the equations. The relations between the grid points have to be established. It is assumed that the value ϕ of a grid point only influences the distribution of ϕ in its immediate neighbours. For this reason, as the number of grid points becomes larger, the numerical solution approaches the real solution of the problem.

There are different methods of discretizing the equations, but the most common ones are exposed in the following lines.

### Finite difference method

The finite difference method (FDM) is based in the Taylor-series expansion. It is used to approximate the derivatives in the differential equation. Taking the three successive points represented in Figure 1, the approximation of the values in the left point (west) and in the right point (east) is easily calculated with Taylor series:

ϕW=ϕP-ΔxdϕdxP+12Δx2d2ϕdx2P-…

ϕE=ϕP+ΔxdϕdxP+12Δx2d2ϕdx2P+…

Using a second order approximation and combining both expressions, it can be easily obtained:

dϕdxP≈ϕE-ϕW2Δx

d2ϕdx2P≈ϕW+ϕE+2ϕPΔx2

These expressions are substituted in the differential equation to obtain the finite-differential equation. This approach is very simple, but it is not used in complex geometries. It also does not enforce the conservation, as it is simply a mathematical approach, which may lead to some problems.

Δx

Δx

P

W

E

Figure 1: Three successive grid points

### Finite volume method

The finite-volume method (FVM) is more used than the FDM. It consists in dividing the domain in different control volumes, so that each control volume surrounds one grid point. Then, the differential equation is integrated over each control volume, ensuring that each of them satisfy the conservation.

P

W

E

S

Qw

Qe

Qn

Qs

Figure 2: Control volume (2D)

## Time integration

Time is a one-way variable, which means that the unknowns only depend on the values in the previous instant of time, and do not depend on the values in the next instant of time. Taking this property into account, to obtain the results of an unsteady problem, the method is to discretize the time and calculate the values for each time step. When the unknowns of one time step are obtained, the calculation moves on to the next time step.

| Qn Qn+1 tn tn+1 Figure 3: Explicit method (β=0) | Qn Qn+1 tn tn+1 Figure 4: Implicit method (β=1) | Qn Qn+1 tn tn+1 Figure 5: Crank-Nicholson (β=0,5) |
| --- | --- | --- |

Time integration can be done using different methods. The ones that are widely used are:

- Explicit method: The simplest method. All the terms are evaluated using the known values of the previous time step tn. It is a first order approximation and easy to compute, but it requires very small time steps in order to achieve convergence.

- Implicit method: It is a very stable first order approximation, useful in problems with large time steps. The terms are evaluated with the values in the next instant of time tn+1.

- Crank-Nicholson: It is a second order approximation. The terms are evaluated using the values of the previous and the next time step.

## Evaluation of the convective term

There are different schemes to evaluate the convective term of the convection-diffusion equation. This term is evaluated in the faces, not in the nodes, so it is necessary to know the value in the faces of the control volume. Some of the most common schemes are listed below1,2, all of them studied in a one-dimensional case to simplify their explanation.

### The central differencing scheme (CDS)

It is the most natural scheme. It is a linear interpolation between the two nearest nodes:

ϕe-ϕP=dPedPEϕE-ϕP

However, CDS is only valid in cases with low Reynolds number. It is a second order approximation, but may produce oscillatory solutions.

### The upwind scheme (UDS)

It assumes that the value of ϕ in the interface is equal to the value of ϕ at the node on the upwind side of the face.

ϕe=ϕP	if	me>0

ϕe=ϕE 	if	me<0

The solutions of the UDS will always be physically realistic, but they may not be completely accurate because it is a first order approximation. However, this method is widely used because of its stability.

### The exponential scheme (EDS)

Taking the generic convection-diffusion equation and assuming a steady one-dimensional problem with a constant Γ and no source term, the analytic solution of the equation is an exponential function:

ϕ-ϕ0ϕL-ϕ0=exp⁡Px/L-1exp⁡P-1

Where ϕ0 and ϕL are the values of the function at x=0 and x=L respectively and P is the Péclet number, a non-dimensional number:

P≡ρuLΓ

In the EDS, this analytic solution is used to determine the value on the faces, using the following expression:

ϕe-ϕP=expPedPe/dPE-1expPe-1ϕE-ϕP

Though this solution is exact for the steady one-dimensional problem it is not for two or three-dimensional cases, unsteady problems… so it is not widely used.

### The hybrid scheme (HDS) and the power-law scheme (PLDS)

Both methods are an approximation of the exponential function used in the EDS. Since exponentials are expensive to compute, the HDS and the PLDS are meant to provide a good result but with simpler functions. They divide the function given by the EDS in different parts and approximate the solution with simpler functions.

# Four materials

The four materials problem is a two-dimensional transient conduction problem. It consists in a long rod composed of four different materials with different properties. The general scheme of the problem is represented in Figure 6:

Figure 6: General scheme of the four materials problem

|  | x [m] | y [m] |
| --- | --- | --- |
| p1 | 0.50 | 0.40 |
| p2 | 0.50 | 0.70 |
| p3 | 1.10 | 0.80 |

Table 2: Problem coordinates

|  | ρ [kg/m3] | cP [J/kgK] | λ [W/mK] |
| --- | --- | --- | --- |
| M1 | 1500.00 | 750.00 | 170.00 |
| M2 | 1600.00 | 770.00 | 140.00 |
| M3 | 1900.00 | 810.00 | 200.00 |
| M4 | 2500.00 | 930.00 | 140.00 |

Table 3: Physical properties of the materials

| Cavity wall | Boundary condition |
| --- | --- |
| Bottom | Isotherm at T=23.00 ºC |
| Top | Uniform Qflow=60.00 W/m length |
| Left | In contact with a fluid at Tg=33.00 ºC and heat transfer coefficient 9.00 W/m2K |
| Right | Uniform temperature T=8.00+0.005t ºC (where t is the time in seconds) |

Table 4: Boundary conditions

The initial temperature field is T=8.00 ºC.

## Conduction equation

The conduction heat transfer is described by Equation 1:

ρcp∂T∂t=∇·λ∇T+qv

Equation 1: Conduction equation

Where ρ is the density, T the temperature, λ the conductivity of the material, cP the specific heat of the material and qv the inner heat of the material (source term).

## Discretization

The domain is discretized using the node centred distribution, to avoid having conflictive control volumes between the different materials. Since it is a transitory problem, it is necessary to discretize in space and time. The method used to discretize the equation is the finite volume method.

### Space discretization

Figure 7: Heat fluxes through the faces of a control volume

The heat fluxes through the walls represented in Figure 7 are integrated:

Qe=-Seλ∂T∂xdS≈-λ∂T∂xeSe≈-λeTE-TPdPESe

Qw=-Swλ∂T∂xdS≈-λ∂T∂xwSw≈-λwTP-TWdPWSw

Qn=-Snλ∂T∂ydS≈-λ∂T∂ynSn≈-λnTN-TPdPESn

Qs=-Ssλ∂T∂ydS≈-λ∂T∂ysSs≈-λsTP-TSdPSSs

Equation 2: Discretized heat fluxes through the walls

Where T is the temperature at the given node, d the distance between two nodes, and λ the conductivity at the given face.

However, since there are different materials there are faces in which the conductivity can have two values: one on the left side of the face and the other one on the right side of the face. This problem is solved with the harmonic mean. It can be calculated with the heat fluxes through the wall:

qe-=qe+

-λPTe-TPdPe=-λETE-TedEe

qe=-λeTE-TPdPE

λe=dPEdPeλP+dEeλE

Equation 3: Harmonic mean

The inner heat of the material can be discretized as:

QVP=VPqvdV≈qvPVP

Equation 4: Discretized inner heat flux (source term)

### Time discretization

The time discretization is done using the First law of thermodynamics:

VPρ∂u∂tdV=QP

Where u is the internal energy of the control volume.

Assuming an incompressible material, the First law of thermodynamics is integrated over time. Taking n as the previous instant of time and n+1 the instant of time that is going to be calculated:

tntn+1ρP∂uP∂tVPdt=tntn+1QPdt

Rearranging the first term of the equation:

tntn+1ρP∂uP∂tVPdt=ρPVPuPn+1-uPn≈ρPVPuPn+1-uPn=ρPVPcPTPn+1-TPn

And the second term:

tntn+1QPdt=βQPn+1+1-βQPnΔt

The discretized equation is finally obtained:

ρPVPcPTPn+1-TPnΔt=β-λwTP-TWdPWSw+λeTE-TPdPESe-λsTP-TSdPSSs+λnTN-TPdPESn+qvPVPn+1+1-β-λwTP-TWdPWSw+λeTE-TPdPESe-λsTP-TSdPSSs+λnTN-TPdPESn+qvPVPn

Equation 5: Discretized conduction equation

To simplify the equation, it can be rewritten with coefficients, dependant on the properties of the nearest nodes in the following form:

aPTP=aETE+aWTW+aNTN+aSTS+bP

The coefficients are called discretization coefficients, and they are different for each node. The discretization coefficients are:

aE=βλeSedPE

aW=βλwSwdPW

aN=βλnSndPN

aS=βλsSsdPS

aP=aE+aW+aN+aS+ρPVPcP/Δt

bP=ρPVPcPTPnΔt+βqvPn+1VP+1-β-λwTP-TWdPWSw+λeTE-TPdPESe-λsTP-TSdPSSs+λnTN-TPdPESn+qvPVPn

Equation 6: Discretization coefficients for the four materials problem

### Boundary conditions

The outer walls of the rod have special conditions, so each of them has to be studied in order to determine the coefficients of the boundary nodes.

In the left wall (i=1), there is convection, so some coefficients have to be recalculated:

aW=0

aP=aE+aW+aN+aS+ρPVPcPΔt+β1α+dPwλP

bP=ρPVPcPTPnΔt+βqvPn+1VP+Tg1α+dPwλP+1-βTg-TP1α+dPwλP+λeTE-TPdPESe-λsTP-TSdPSSs+λnTN-TPdPESn+qvPVPn

Equation 7: Discretization coefficients of the left wall

In the top wall (j=1) there is a constant heat flux. The new coefficients are:

aN=0

bP=ρPVPcPTPnΔt+βqvPn+1VP+QflowSnStop+1-β-λwTP-TWdPWSw+λeTE-TPdPESe-λsTP-TSdPSSs+qvPVPn

Equation 8: Discretization coefficients of the top wall

In the right wall (i=N), the temperature Tr is given, and it changes over time. The coefficients are very similar to those of the general case. The only differences are:

aE=0

aP=aE+aW+aN+aS+ρPVPcPΔt+βλPSedPe

bP=ρPVPcPTPnΔt+βqvPn+1VP+λPSedPeTrn+1+1-β-λwTP-TWdPWSw+λPTr-TPdPeSe-λsTP-TSdPSSs+λnTN-TPdPESn+qvPVPn

Equation 9: Discretization coefficients of the right wall

Finally, in the bottom (j=M) the temperature Tb is also given, but it is constant. The approach is very similar to that of the right wall. So that the coefficients are:

aS=0

aP=aE+aW+aN+aS+ρPVPcPΔt+βλPSsdPs

bP=ρPVPcPTPnΔt+βqvPn+1VP+λPSsdPsTb+1-β-λwTP-TWdPWSw+λeTE-TPdPESe-λPTP-TbdPsSs+λnTN-TPdPESn+qvPVPn

Equation 10: Discretization coefficients of the bottom wall

## Algorithm

YES

YES

Tn+1*=Tn+1

Input data:

ρ, cP, λ, α, Tg, Tr, Tb, Qflow, δ, β…

Previous calculations:

x, y, S, V…

Initial temperature map:

tn=0; T=T0

Calculation of the next time step:

tn+1=tn+Δt; Tn+1*=Tn

Evaluation of the constant coefficients:

aE, aW, aN, aS, aP

Solve the set of equations:

Tn+1

Final calculations and print results

END

maxTn+1-Tn+1*<δ?

New time step?

NO

NO

Evaluation of the non-constant coefficients:

bP

## Results

Figure 8: Instantaneous isotherms at t=5000 s

Figure 9: Instantaneous results at t=10000 s

# Smith-Hutton problem

The following problem is a two-dimensional steady convection-diffusion problem, represented in Figure 10:

Figure 10: General scheme of the Smith-Hutton problem

The velocity field is given by:

ux,y=2y1-x2

vx,y=-2x1-y2

Equation 11: Prescribed velocity

And the boundary conditions for the variable ϕ are:

ϕ=1+tanhα2x+1               y=0 ;x∈-1,0  (inlet)∂ϕ∂y=0               y=0 ;x∈0,1  (outlet)ϕ=1-tanhα               (elsewhere)

Equation 12: Boundary conditions of the Smith-Hutton problem

Where α=10.

## General convection-diffusion equation

The mathematical formulation needed to solve this problem is the convection diffusion-equation:

∂ρϕ∂t+∇·ρvϕ=∇·Γ∇ϕ+Sϕ

Equation 13: Generic convection-diffusion equation

## Discretization

It is necessary to discretize Equation 13 in space and time. The control volume used to discretize the problem is specified in Figure 11. The boundary nodes are in blue and the inner nodes in black.

Figure 11: Mesh of the Smith-Hutton problem

To do so, it is easier to start with the discretization of the mass equation:

∂ρ∂t+∇·ρv=0

Equation 14: Mass conservation equation

The first step is to integrate Equation 14 over time and space. Taking only the first term of the equation:

tntn+1VP∂ρ∂tdVdt=tntn+1Vp∂ρp∂tdt=Vpρpn+1-ρpn≈Vpρpn+1-ρpn=Vpρp-ρp0

Using the Divergence Theorem, the second term of the equation transforms into a surface integral:

tntn+1VP∇·ρvdVdt=tntn+1Sfρv·ndSdt=tntn+1me-mw+mn-msdt≈men+1-mwn+1+mnn+1-msn+1∆t

In the second term, to integrate over time an implicit scheme is used. The final discretized mass equation is:

Vpρp-ρp0∆t+me-mw+mn-ms=0

Equation 15: Discretized mass equation (2D)

The discretization of the convection-diffusion equation is very similar to that of the mass equation. Integrating over the volume the transport term of Equation 13 and applying the Divergence Theorem:

VP∇·ρvϕdV=Sfρϕv·ndS=meϕe-mwϕw+mnϕn-msϕs

The same procedure is used for the diffusion term:

VP∇·Γ∇ϕdV=SfΓ·∇ϕ·ndS=-Γw∂ϕ∂xwSw+Γe∂ϕ∂xeSe-Γs∂ϕ∂ysSs+Γn∂ϕ∂ynSn≈DeϕE-ϕP-DwϕP-ϕW+DnϕS-ϕP-DsϕP-ϕS

Where:

De=ΓeSedPE

Dw=ΓwSwdPW

Dn=ΓnSndPN

Ds=ΓsSsdPS

To simplify the source term, it is linearized:

VPSϕdV≈Sϕ,PVP=Scϕ+SpϕϕPVP

So that the resulting equation is:

ρPϕP-ρP0ϕP0∆tVP+meϕe-mwϕw+mnϕn-msϕs=DeϕE-ϕP-DwϕP-ϕW+DnϕS-ϕP-DsϕP-ϕS+Sϕ,PVP

Multiplying by ϕ the discretized mass equation (Equation 15) and subtracting the result in this equation, the discretized convection-diffusion equation is obtained:

ρP0ϕP-ϕP0ΔtVP+meϕe-ϕP-mwϕw-ϕP+mnϕn-ϕP-msϕs-ϕP=DeϕE-ϕP-DwϕP-ϕW+DnϕS-ϕP-DsϕP-ϕS+Scϕ+SpϕϕPVP

To simplify the problem, a new variable is introduced to the problem: the total flux. But this variable is split in the two dimensions of the problem, the flux in the x-direction and the flux in the y-direction.

Jx≡ρuϕ-Γ∂ϕ∂x

Jy≡ρvϕ-Γ∂ϕ∂y

Equation 16: Total (convection plus diffusion) fluxes

Introducing the expressions of the total flux, the discretized convection diffusion equation becomes:

ρP0ϕP-ϕP0ΔtVP+Je-FeϕP-Jw-FwϕP+Jn-FnϕP-Js-FsϕP=Scϕ+SpϕϕPVP

Equation 17: Final discretization of the convection diffusion equation

Where the flow rates are:

Fe=ρueSe

Fw=ρuwSw

Fn=ρvnSn

Fs=ρvsSs

However, it is necessary to know how the fluxes are going to be evaluated. In order to use non-dimensional numbers, a new variable is defined:

J*≡JδΓ=Pϕ-dϕdx/δ

Where P is the Péclet number and δ is the distance between the point that is going to be studied i, and the point next to it, i+1. The value of φ and the value of the gradient dϕ/d(x/δ) are a combination of the ϕi and ϕi+1, so that J* can be expressed as:

J*=Bϕi-Aϕi+1

The coefficients A and B are dimensionless and depend on the Péclet number. However, since B is a combination of A and the Péclet number, and both coefficients are symmetric, it can be deduced that:

AP=AP+-P,0

BP=AP+P,0

Equation 18: Dimensionless coefficients A and B

Where A,B is an operator that denotes the greater of A and B.

Combining Equation 17 and Equation 18, the following formulation is obtained:

aPϕP=aEϕE+aWϕW+aNϕN+aSϕS+bP

Where

aE=DeAPe+-Fe,0

aW=DwAPw+Fw,0

aN=DnAPn+-Fn,0

aS=DsAPs+Fs,0

aP=aE+aW+aN+aS+ρP0VPΔt-SPVP

bP=SCVP+ρP0VPΔtϕP0

Equation 19: Discretization coefficients of the Smith-Hutton problem

And the Péclet numbers are:

Pe=FeDe

Pw=FwDw

Pn=FnDn

Ps=FsDs

The only operation that should be defined is the value of the coefficient A. This value depends on the integration scheme that is going to be used. Some of its values are listed in Table 5:

| Scheme | Formula for AP |
| --- | --- |
| Central difference (CDS) | 1-0.5P |
| Upwind (UDS) | 1 |
| Hybrid (HDS) | 0,1-0.5P |
| Power law (PLDS) | 0,1-0.1P5 |
| Exponential (EDS) | P/expP-1 |

Table 5: Function AP for different schemes1

## Boundary conditions

To apply the boundary conditions to the problem, some modifications on the coefficients defined by Equation 19 have to be done. The easiest one is for the points in the left, top and right sides of the domain, in which the value of ϕ is defined.

In the bottom it is necessary to distinguish between the inlet and the outlet. A similar approach to that of the left, top and right side is used to determine the coefficients of the points in the inlet. However, in the outlet the only condition is that the derivative of ϕ in the vertical direction is equal to zero. The following reasoning leads to:

∂ϕ∂y≈ϕN-ϕPΔy=0

ϕP=ϕN

The implementation of the boundary conditions in the problem is done with the discretization coefficients listed in Table 6.

| Coefficients | Left, top and right | Inlet (bottom) | Outlet (bottom) |
| --- | --- | --- | --- |
| aE | 0 | 0 | 0 |
| aW | 0 | 0 | 0 |
| aN | 0 | 0 | 1 |
| aS | 0 | 0 | 0 |
| aP | 1 | 1 | 1 |
| bP | 1-tanhα | 1+tanhα2x+1 | 0 |

Table 6: Discretization coefficients of the boundary points

## Algorithm

## Results

The computation of the results has been done using different schemes. In order to compare them with the reference solutions, some plots have been done.

Figure 12: ρ/Γ=10

Figure 13: ρ/Γ=103

Figure 14: ρ/Γ=106